CACHE / AIChE Modules on Energy in the Curriculum

Module Title: Solar Panel Energy Analysis

Module Author: Jason Keith

Author Affiliation: Michigan Technological University

Concepts: Numerical Integration; Electrolysis

Problem Motivation: The availability of energy has become an important part of our society. In this and related problems, we will discuss issues of energy consumption, energy reserves, and energy related emissions. Furthermore, we will analyze conventional and alternative energy systems. A particular emphasis will be placed on the generation of hydrogen for use in fuel cells for transportation and/or stationary applications.

Problem Information

Example Problem Statement: During a sunny day, a solar photovoltaic array produces the following instantaneous amounts of electrical power.

| Time | Power (W) | Time | Power (W) | Time | Power (W) | Time | Power (W) | Time | Power (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6:00 | 0 | 9:15 | 87 | 12:30 | 186 | 15:45 | 164 | 19:00 | 20 |
| 6:15 | 0 | 9:30 | 99 | 12:45 | 189 | 16:00 | 157 | 19:15 | 20 |
| 6:30 | 2 | 9:45 | 110 | 13:00 | 192 | 16:15 | 152 | 19:30 | 19 |
| 6:45 | 4 | 10:00 | 120 | 13:15 | 194 | 16:30 | 142 | 19:45 | 11 |
| 7:00 | 7 | 10:15 | 129 | 13:30 | 195 | 16:45 | 131 | 20:00 | 5 |
| 7:15 | 9 | 10:30 | 138 | 13:45 | 194 | 17:00 | 124 | 20:15 | 4 |
| 7:30 | 13 | 10:45 | 147 | 14:00 | 193 | 17:15 | 110 | 20:30 | 3 |
| 7:45 | 21 | 11:00 | 155 | 14:15 | 192 | 17:30 | 93 | 20:45 | 2 |
| 8:00 | 27 | 11:15 | 163 | 14:30 | 190 | 17:45 | 59 | 21:00 | 0 |
| 8:15 | 38 | 11:30 | 168 | 14:45 | 186 | 18:00 | 20 |  |  |
| 8:30 | 49 | 11:45 | 172 | 15:00 | 180 | 18:15 | 12 |  |  |
| 8:45 | 61 | 12:00 | 178 | 15:15 | 177 | 18:30 | 26 |  |  |
| 9:00 | 73 | 12:15 | 183 | 15:30 | 155 | 18:45 | 23 |  |  |

- Determine the cumulative electrical energy produced in kW-hr

- Determine the cumulative electrical energy produced in J

- An electrolysis unit operates at 60% efficiency. If the theoretical amount of energy needed to convert 1 mol of liquid water into 1 mol of hydrogen and ½ mol of oxygen is 285 kJ, determine the mass of hydrogen produced in g for one day.

- If a fuel cell car can travel 60 miles per kg of hydrogen, determine the driving range from the solar power.

Example Problem Solution:

- We can use numerical integration using the Trapezoidal rule to determine the total amount of electrical energy produced by the panel.

(1)

For a constant step size t we can calculate the energy production as:

(2)

In our data, the step size is uniform at ¼ hr so we can use equation 2 to estimate the energy production. We obtain:

E = (½ * 0 + ½ * 0 + 0 + 2 + 4 + 7 + …) W (1/4) hr = 1468 W-hr = 1.47 kW-hr

- We can convert units to calculate the energy production in J.  We have:

(3)

- We can convert units to calculate the hydrogen production in mol, and use the molecular weight to give the number of grams of hydrogen.  Noting the 60% efficiency, we have:

(4)

- We can use the fuel economy of the fuel cell vehicle to estimate the driving distance.

Home Problem Statement: During a cloudy day, a solar photovoltaic array produces the following instantaneous amounts of electrical power.

| Time | Power (W) | Time | Power (W) | Time | Power (W) | Time | Power (W) | Time | Power (W) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6:00 | 0 | 9:15 | 22 | 12:30 | 17 | 15:45 | 155 | 19:00 | 29 |
| 6:15 | 0 | 9:30 | 29 | 12:45 | 6 | 16:00 | 149 | 19:15 | 12 |
| 6:30 | 0 | 9:45 | 108 | 13:00 | 3 | 16:15 | 146 | 19:30 | 19 |
| 6:45 | 0 | 10:00 | 117 | 13:15 | 20 | 16:30 | 138 | 19:45 | 16 |
| 7:00 | 1 | 10:15 | 121 | 13:30 | 121 | 16:45 | 128 | 20:00 | 16 |
| 7:15 | 1 | 10:30 | 142 | 13:45 | 170 | 17:00 | 119 | 20:15 | 12 |
| 7:30 | 1 | 10:45 | 100 | 14:00 | 182 | 17:15 | 109 | 20:30 | 9 |
| 7:45 | 2 | 11:00 | 64 | 14:15 | 170 | 17:30 | 99 | 20:45 | 7 |
| 8:00 | 3 | 11:15 | 84 | 14:30 | 102 | 17:45 | 86 | 21:00 | 0 |
| 8:15 | 5 | 11:30 | 92 | 14:45 | 24 | 18:00 | 75 |  |  |
| 8:30 | 13 | 11:45 | 59 | 15:00 | 56 | 18:15 | 63 |  |  |
| 8:45 | 31 | 12:00 | 78 | 15:15 | 177 | 18:30 | 51 |  |  |
| 9:00 | 12 | 12:15 | 27 | 15:30 | 164 | 18:45 | 39 |  |  |

- Determine the cumulative electrical energy produced in kW-hr

- Determine the fraction of energy produced compared with the sunny day from the example problem.

- An electrolysis unit operates at 60% efficiency. If the theoretical amount of energy needed to convert 1 mol of liquid water into 1 mol of hydrogen and ½ mol of oxygen is 285 kJ, determine the mass of hydrogen produced in g for one day.

- If each solar panel has area of 70” x 40”, determine the number of panels and solar surface area in ft2 required to supply 1 kg of hydrogen on cloudy days.